## Finite Element Theoretical Background

Prof. E. Mucchi

## REFERENCES

G. Diana, F. Cheli, Dinamica e vibrazione dei sistemi, 1993.

C. De Silva, Computer techniques in Vibration, 2007.

S. Rao, Mechanical Vibrations, 1995.

M. Petyt, Introduction to finite element vibration analysis, 1990.

C. Gianini, La progettazione strutturale con il calcolatore, 2003.

L. Meirovitch, Fundamentals of vibrations, 2001.

W. Heylen, S. Lammens, P. Sas, Modal analysis theory and testing, 2004

# Numerical Methods for Vibratory Analysis

## Introduction [Diana-Cheli, Petyt, Rao, De Silva]

Real systems may be represented as continuous systems with an infinite number of degrees of freedom (DOFs).  By using the continuum theory, even the behaviour of simple systems is governed by a set of complex partial differential equations (e.g. axial, torsional, flexural vibrations of a beam). Furthermore, real systems are often complex in shape, they may be made of different materials or be subjected to complicated loading histories (e.g. vehicles, machine components, etc…). In these conditions, analytically obtaining the equations of motion which satisfy the boundary conditions effectively applied on the system is impossible. Therefore, approximated techniques have been developed in order to solve this problem.

Usually, continuous systems with infinite DOFs are discretized in an approximated model with a finite number (N) of DOFs, which must be able to adequately replicate the same behaviour.

Various discretization techniques exist, such as:

- Lumped Parameter method (theory of 1 DOF and N DOFs systems);

- Finite Element Method (FEM);

- Boundary Element Method (BEM);

- Multibody method.

For lumped parameter systems with N DOFs, it is possible to obtain the natural frequencies and the modal shapes in their exact form by equating the determinant of the characteristic equation to zero and solving the resulting system of coupled equations.

For large values of N, the solution of the system may take a long time: in these cases, it is possible to use the modal method to obtain an uncoupled system of the same order of magnitude; alternatively, natural frequencies and mode shapes can be assessed by means of numerical or analytical methods, such as: Dunkerley’s formula, Rayleigh’s quotient, Holzer’s method, matrix iteration methods, Jacobi’s method.

In particular, the Dunkerley’s formula allows to estimate the square of the first natural frequency :

where  and  are the coefficients on the main diagonal of the compliance and mass matrix, respectively.

Similarly, the Rayleigh’s quotient is used to assess the value of :

where represents the mode shape of the j-th mode, while  and  are the stiffness and mass matrixes, respectively. Usually, the Rayleigh’s quotient is employed to estimate the first natural frequency: in fact, the mode shape of the first mode is generally simple, hence it can be easily obtained. Furthermore, the dynamic behaviour of most structures is mainly governed by the first modes: the estimation of the first natural frequency, then, is of great importance. It must be noted that the accuracy of the estimation of the first natural frequency depends on the accuracy of the assumed first mode.

In order to calculate the complete solution for N DOFs systems or for continuous systems, numerical methods have been developed. By employing these techniques, the solution is not limited to a few natural frequencies and mode shapes.

All the numerical integration methods for the differential equations of motion share the same two characteristics:

- they do not satisfy the equations of motion at every instant t, but only at discrete time intervals ();

- they assume a possible type of variation for the displacement, the velocity or the acceleration, usually by means of the Taylor series decomposition (finite difference method, Runge-Kutta method, Houbolt’s method, Wilson’s method, Newmark’s method) or arbitrarily assuming possible mode shapes (Ritz-Rayleigh’s method, finite element method).

## 2. Formulation of the equations of motion using an energetic approach (Hamilton’s principle)

The first step needed to analyse the vibratory behaviour of a mechanical system is to correctly write the equations of motion. In this respect, more or less automated methods exist:

- method based on the Newton’s law;

- method based on the principle of virtual work:

- method based on the D’Alembert’s principle;

- compliance method;

- method used by the Multibody techniques;

- influence coefficients method;

- method based on the Hamilton’s principle.

Below, a method which forms the basis of the finite element method will be described: a technique based on an energetic approach which exploits the Hamilton’s principle. The principle states that “the integral between t1 and t2 of the variation of kinetic energy  and work made by internal and external forces is equal to zero”:

where  is the sum of the variation of works due to conservative and non-conservative forces:

In particular,  is due to the dissipative forces (e.g. damping) or to forces which transfer energy to the system (e.g. external forces), while  is defined as the inverse of the variation of potential elastic energy:

Substituting Eq.  and Eq.  on Eq. :



The advantage of this formulation is that energetic quantities are used: complexities associated to vectors are removed, because energetic terms are scalar.

For instance, for the 1 DOF system depicted in Figure 1, the kinetic energy T, the potential energy U and  are:

; ;

Substituting Eq.  on Eq. , the standard formulation of the equation of motion is obtained:

.

The mathematical procedure is reported in Petyt pp. 9-10.

The Hamilton’s principle   applied to a N DOFs discrete system may be written using the Lagrange’s equations (some mathematical passages are shown in Petyt page 10):

where:

- independent generalized coordinates of the N DOFs system;

- kinetic energy, function of velocity ;

- elastic potential energy, function of displacements ;

- variation of the work made by non-conservative forces;

- dissipation function which depends from velocities ;

The quantities T, D and U may be systematically computed using the following expressions:

where  are the mass, damping and stiffness matrix of the N DOFs system, while the column vector is the generalized coordinates vector:

It must be noted that, in the Lagrange’s equations , the first addend is the kinetic energy term which we demonstrate to be equal to :

in fact, for a 1 DOF system, the kinetic energy T is defined in , thus:

In analogy, it can be proved that the second term in the Lagrange’s equations is the dissipative term, while the third is related to the stiffness matrix:

Therefore, the Lagrange’s equations  may be traced back to the standard formulation of the equations of motion for N DOFs systems:

Figure 1. 1 DOF system.

## 3. The Ritz-Rayleigh method

For real systems, the solution of the Hamilton’s equation is complex, and the closed-form solution may not exist. As a result, approximated methods have been created. One of them is the Ritz-Rayleigh’s method (R.R.) which approximates the solution with an expansion in the form

where  are time-dependent unknown functions which will be derived solving an eigenvalues – eigenvectors problem, while  are the prescribed functions known a priori, which represent a possible deformed configuration of the system (R.R. form functions), and m is the considered order, which means the number of terms considered in the summation.

The R.R. method for the study of free vibrations (no external forces) and undamped systems consists of the following steps:

- Choose a function (named v(x,t)) which is an approximated solution of the Hamilton’s equation  that satisfies the boundary conditions of the problem;

- Compute the potential elastic energy;

- Compute the kinetic energy;

- Use the Lagrange’s equations to derive the equation of motion;

- Solve an eigenvalues – eigenvectors problem. In particular, solving the characteristic equation ) the unknowns (eigenvectors) and natural frequencies  (eigenvalues) are obtained. The eigenvectors allow to determine the unknown functions  found in Eq.  using the expression:

.

The unknown functions are defined at the natural frequencies , and they are as many as them. Then, the mode shape is obtained using Eq. :

Eventually, the free motion at the natural frequency  can be written as:

Hereinafter, only the flexural vibrations of a beam will be considered (Figure 2), hence the expressions to be used for T and U will be:

Figure 2. Reference frame for a beam subjected to flexure.

The function v(x,t) is peculiar. It is an expansion formed by two functions, one dependent by the time only, and the other by the space only.  The form functions  are arbitrarily chosen, however they must meet the following conditions:

- They must be linearly independent;

- They must be p-times differentiable, where p is highest order appearing in the potential elastic energy expression (p = 2 in our case);

- They must satisfy the boundary conditions;

- They must form a complete series, which means that the limit of the mean quadratic error must be zero:

Usually the form functions  are:

- Polynomial functions: ;

- Trigonometric functions: ;

- Legendre’s, Jacobi’s, etc… polynomials;

EXAMPLE 1. Flexural vibrations of a cantilever beam.

Determine the first two natural frequencies, assuming that the Young’s modulus (E), the density (), area and moment of inertia of the section (A, Iz) and length (l) are known.

Let  be the prescribed form function, so that the deformation is:

Figure 3. First and second mode shapes obtained with a two-terms R.R. function.

Therefore, by increasing the number of terms considered in the expansion of R.R., greater accuracy in the estimation of the natural frequencies is achieved, and the number of assessed natural frequencies increases.

Exercise (needed for the exam, during the oral examination). Flexural vibrations of a steel cantilever beam. Determine the first two natural frequencies and the corresponding mode shapes (with the first component of each mode equal to 1) for a beam of length 3 meters and section dimensions:

h = 4 cm, t = 3 mm and s = 3 mm.

Assume

h

h

Solve the exercise in written form, deriving the modes and the natural frequencies analytically: then, solve the exercise in the Matlab environment, showing the plots of the mode shapes.

Example of solution in the Matlab environment

- Matrix M e K definition

M = [……;

….

- Computation of frequencies and modes

[fi,omegaq] = eig(K,M);

for i =1:2,

finorm(:,i) = fi(:,i)./fi(1,i);

end;

omega=sqrt(omegaq)

%%%modes

x=0:0.01:L;

v1=4*(x/L).^2+finorm(2,1).*2*(x/L).^3;

v2=4*(x/L).^2+finorm(2,2).*2*(x/L).^3;

- Mode shapes plots

figure

plot(x,v1)

## 4. The Finite Element Method

Basically, the R.R. method is a discretization technique used to derive approximate solutions of the equation of motion of the system when the displacement v(x,t) is obtained as a linear combination of the prescribed functions  multiplied by the unknown functions .

The latter are derived solving an eigenvalue problem. Anyway, we should be asking when the solution of this problem (v(x,t)) correctly approximates the differential equation of motion of the continuous system.

The degree of accuracy of the R.R. method lies mainly in the quality of the prescribed form functions and their number, as seen in the previous example. However, for complex systems it may be difficult to define a priori a possible form function on the entire domain. In fact, in the R.R. method the form function must be defined on the entire domain of the structure (in the previous example, between 0 and L). The finite element method allows overcoming this problem, since the form functions are defined on small subdomains of the complete system, which are named finite elements. The set of finite elements is called mesh. Usually these functions are low-order polynomials and are the same for every finite element. The procedure followed by finite element methods is the following:

- divide the structure in a certain number of elements with finite dimension. The elements are connected to each other by nodes;

- associate a certain number of DOFs to each node;

- create a set of functions (form functions) so that each one has unit value in a degree of freedom and zero in the others;

- substitute the form functions of an element in the expressions of the kinetic and the potential energy to obtain the mass and stiffness matrices for each finite element;

- sum the kinetic and potential energies for each element to obtain the energies for the complete system (assembling of mass and stiffness matrices);

- impose the boundary conditions;

- solve the problem (e.g. an eigenvalues – eigenvectors problem to obtain modes and natural frequencies)

.

Figure 4. Importance of CAE in the design process.

Figure 5. Origin of FEM.

Figure 6. Variability in real structures.

Figure 7. Competition among prestigious universities during the eighties.

Some interesting links:

http://www.youtube.com/watch?v=U9swU5J3gLI&NR=1

http://www.youtube.com/watch?v=L3x5iq3oT9U&feature=related

http://www.youtube.com/watch?v=_NHqXWohW2g&feature=related

http://www.youtube.com/watch?v=0NG9v3JdfEs&feature=related

### 4.1 Theoretical discussion for the beam (see Petyt)

Below, the finite element method applied to the case of free flexural vibrations of a beam will be shown (non-forced vibrations). The methodology described in the previous paragraph will be explained in detail.

- Divide the structure in a certain number of elements with finite dimension. The elements are connected to each other by nodes.

Figure 8. Beam with uniform constant section discretized with 4 elements.

- Associate a certain number of DOFs to each node.

To study the flexural vibration of the beam, a displacement along y (v) and a rotation around the z axis () have been associated to each node.

Figure 9. Rotational and flexural degrees of freedom associated to the five nodes.

- Create a set of functions (form functions) so that each one has unit value in a degree of freedom and zero in the others (Figure 10).

v1

v2

v3

v4

θz1

θz2

θz3

θz4

Figure 10. Element form functions for flexural vibrations of the beam.

In analogy with the R.R. method, the form functions must satisfy the following conditions:

- They must be linearly independent;

- They must be continuous functions p-times differentiable inside the element, where p is highest derivation order which appears in the expression of the potential energy;

- If the form functions are polynomials, they must be complete polynomials of order p or higher; they may be incomplete, but only if their order is higher than p.

- They must satisfy the boundary conditions.

To study the flexural vibrations of a beam, a complete polynomial of order 3 has been chosen. In fact, the highest derivation order appearing in Eq.  is 2.

Consider a beam finite element with density, Young’s modulus E, inertia and section area Iz and A (Figure 11); shear effects and rotational inertia are neglected.

Figure 11. Notation used to study the flexural vibrations a beam element.

The third order polynomial which describes the deformed configuration of the finite element as a function of the dimensionless coordinate is:

Which can be rewritten in matrix form:

To obtain an expression similar to Eq.  but referred to the rotational degree of freedom, it is necessary to recall that  and differentiate Eq. :

And multiplying both sides by a:

Eq.  and Eq.  represent the flexural and rotational deformed shapes of the finite element. Evaluating these expressions for both ends ():

In compact form, Eq.  may be rewritten as:

Solving for :

where

Eq.  may be written in a different form:

where  and

Then, by substituting Eq.  in Eq. :

Where

With:

Hence, considering Eq. , the provides the bending or rotation values for each degree of freedom of the system. The difference between Eq.  and Eq.  is that Eq.  is specified for the DOFs of the finite element.

- Substitute the form functions of an element in the expressions of the kinetic and the potential energy to obtain the mass and stiffness matrices for each finite element.

According to Eq.  the kinetic energy of an element is defined as:

Once  is computed, it can be substituted into Eq. :

The element kinetic energy may be also defined using Eq. :

Thus, comparing Eq.  with Eq. , the mass matrix of the element becomes:

The same procedure may be repeated to obtain the stiffness matrix of the element, starting from the definition of the elastic potential energy shown in Eq. :

Then,  and  are computed and substituted in Eq. :

The kinetic potential energy may be also defined according to Eq. :

Hence, comparing Eq.  with Eq. , the stiffness matrix of the element becomes:

- Sum the kinetic and potential energies for each element to obtain the energies for the complete system (assembling of mass and stiffness matrices);

Define  as the vector containing all the DOFs of the 4-element beam considered in this example:

This vector can be related to the one containing the DOFs of every single element through the matrix :

e.g. the transformation matrix for the first element is:

The total kinetic energy is the sum of the kinetic energies of each element:

Writing Eq.  up to the second element:

Similarly, the same may be done for the potential energy and the stiffness matrix:

- Impose the boundary conditions.

Geometric boundary conditions may be imposed, in the case of clamps, blocking the involved DOFs. For example, if the beam is clamped at one end,  and  will be equal to zero. As a consequence, the rows and columns of the mass and stiffness matrices corresponding to the blocked DOFs are removed; for example, deleting the highlighted parts of the matrices:

- solve the problem (e.g. an eigenvalues – eigenvectors problem to obtain modes and natural frequencies)

In the R.R. method, the accuracy of the solution was improved increasing the terms of the prescribed functions. In the finite element method, in order to increase the number of prescribed functions (form functions), it is necessary to raise the number of nodes and elements, which means generating a denser mesh. Alternatively, the degree of the polynomial form function may be increased, by selecting a higher-order series.

## Exercise (needed for the exam, during the oral examination) Implement the finite element method for a clamped beam in Matlab.

Implement, in the Matlab environment, the finite element method for the clamped beam showed below. Use 4 elements to model the beam. Show the first 8 natural frequencies and the corresponding mode shapes. Print the Matlab script.

The dimensions of the beam are:

L=0.8m; L1=L2=0.4 m; h1=4cm; h2=2cm; b=3cm; t=3 mm; s=2.5mm.

Example of solution

1) Data input

% Data

n = 4; % Number of elements

data = zeros(9,n);

data(1,:) = [0.03 0.03 0.03 0.03]; % b width of the element section

2)  Definition of the mass and stiffness matrices for the element

Define the formulae for the stiffness and mass matrices of the element (define also an index for the element).

M(i,j,k), where k is the index of the element and i, j are the indexes of the matrix.

3) Assembling

4) Boundary conditions

% Application of the boundary conditions (clamped beam)

M(1:2,:) = [];

K(1:2,:) = [];

M(:,1:2) = [];

K(:,1:2) = [];

5) Solving the eigenvalues problem

[fi,omegaq] = eig(K,M);

freq_nat=sqrt(omegaq)/(2*pi)

6) Modes plot

first_eigenvector = [0 fi(1:2:end,1)']

plot(first_eigenvector), title('primo modo')

Figure 12. First 8 modes for the clamped beam subjected to flexure, modelled with 4 finite elements.

### 4.2 Mass matrix “Lumped” and “consistent”

The mass matrix obtained in Eq.  is named “consistent”, because it is obtained using the same method used to determine the stiffness matrix. However, many problems may be solved using simpler forms of the mass matrix. The easiest formulation is named “lumped”, because the matrix is obtained neglecting the inertial effects and concentrating the entire mass on the nodes of the element. To study the flexural vibration of the beam, where each node has 2 DOFs, half of the mass is concentrated on the left translational DOF, while the other half is applied on the right translational DOF. The resulting mass matrix is:

It is clear that the “consistent” approach provides more accurate results, since it does not neglect the inertial effect and the coupling between the degrees of freedom. However, the “lumped” formulation is widely used, because the resulting mass matrix is diagonal, leading to a large reduction in the computational time.

Figure 13 shows the percentage error for the first 15 natural frequencies of a cantilever beam obtained using the “lumped” and “consistent” formulations. Interestingly, as the number of finite elements increases, the two formulations provide more similar results.

10 elements

50 elements

100 elements

200 elements

Figure 13. Percentage error (compared to the theoretical solution for a continuous system) for the first 15 natural frequencies of a clamped beam modelled with a variable number of elements (10, 50, 100, 200). Purple = “lumped”, light blue = “consistent”.

### 4.3. Techniques used to decrease the number of degrees of freedom (Model reduction)

The mesh generation may generate a large number of elements, that is, a large number of DOFs and a long computational time. In particular, by employing automatic schemes for the mesh generation or using three-dimensional elements, the number of DOFs quickly increases. Many techniques exist which allow to reduce the number of DOFs; among these:

- Model simplification. Remove from the CAD (before meshing) all the parts, which are useless for the analysis (bolts, rivets, small holes, etc…);

- Simplification by idealization. Use, if possible, beam or shell elements rather than 3D elements (e.g. to mesh a beam-shaped structure);

- Exploit the symmetry of the problem.

- Modal reduction. Consider a problem () of dimension N: mass and stiffness matrices will both have a dimension of NxN and the DOFs vector will be Nx1. By applying a coordinate change  where  has dimension mx1 with m<N and  represents the eigenvectors matrix (rectangular matrix in which the eigenvectors corresponding to the highest natural frequencies have been removed), the size of the problem decreases. Notably, mass and stiffness matrices are diagonal, and their dimension is mxm:

- Guyan’s reduction or static reduction. In this procedure, the total number of DOFs of the problem are divided in two groups: master and slave. Slave DOFS are removed and expressed as a function of the master DOFs via a static relation. Then, the equation of motion for a forced undamped system may rewritten using the slave and master DOFs:

The two equations in Eq.  are, then:

It is assumed that the relation between master and slave DOFs is not influenced by mass and inertia (static reduction), therefore their contribution is removed from Eqs.  and :

Solving Eq.  as a function of :

Substituting Eq.  into Eq. :



Which may be rewritten as:

where:

e

However,

By comparing Eq.   with Eq.  the transformation matrix for coordinate reduction W is obtained:

Hence the W matrix, which reduces the number of coordinates (), has been derived. The new equation of motion is then obtained using Eq. .

### 4.4 Structural damping

An accurate analysis of real structures suggests that the viscous damping is not representative of models with many degrees of freedom, such as finite element models. It appears, in fact, that in these cases the damping is related to the inverse of the frequency.

For a 1 DOF system the structural damping is defined as:

,

and the corresponding structural damping force is:

.

The equation of motion for a forced 1 DOF system with structural damping becomes:



By defining a coefficient  named loss factor, the term can be replaced with =, which introduces a complex stiffness term in the equation:

Typical values of  can range from 10-5 for aluminium and steel to 1 for plastics (rubbers).

Eventually, the equation can be generalized for system with many degrees of freedom:

## Exercise (needed for the exam during the oral examination) – Clamped beam in NX Nastran - Simcenter3D

Consider a clamped beam (Figure 14, Figure 15) with rectangular section 0,15x0,44 m and length 10 m. The beam is made of aluminium. The beam must be modelled with CBEAM (1D) elements and CTETRA (3D) elements as in (Figure 14, Figure 15).

Figure 14: clamped beam modelled with CBEAM elements

Figure 15: beam modelled with CTETRA elements

The clamp, in the case of the beam modelled with CBEAM elements, has been obtained restraining the three translations and the three rotations at one end (i.e. one node), while for the beam modelled with CTETRA elements it has been obtained restraining these degrees of freedom for every node on one side of the parallelepiped (Figure 16).

Figure 16: constraints for the beam modelled with CTETRA elements.

It is asked to model the beam with Simcenter3D, at first considering it one-dimensional (CBEAM elements) and later three-dimensional (CTETRA elements). The goal is to compare the numerical natural frequencies (obtained with the SOL103) with the theoretical natural frequencies.

Report the results in the table below (Table 1).

Table 1 - Clamped beam modelled with 1000 CBEAM elements and 1000 CTETRA elements

| Modes | Frequencies BEAM CBEAM [Hz] | Frequencies BEAM CTETRA [Hz] | Mode (indicate the mode type, e.g. flexural, torsional, axial) | Theoretical frequencies [Hz] | % Error Theoretical vs CBEAM | % Error Theoretical vs CTETRA |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |

An example of a .BDF/.DAT file (Nastran input file)

$*$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$*

$* EXECUTIVE CONTROL

$*

$*$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$*

ID,NASTRAN,solution_name

SOL 103

CEND

$*$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$*

$* CASE CONTROL

$*

$*$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$*

ECHO = NONE

SPC = 1

METHOD = 100

OUTPUT

DISPLACEMENT(PLOT,REAL) = ALL

$*  Step: Subcase - Eigenvalue Method 1

SUBCASE 1

LABEL = Subcase - Eigenvalue Method 1

METHOD = 100

OUTPUT

DISPLACEMENT(PLOT,REAL) = ALL

$*

$*$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$*

$* BULK DATA

$*

$*$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$*

BEGIN BULK

$*

$* SOLUTION CARDS

$*

$*  Modeling Object: Real Eigenvalue - Lanczos1

EIGRL        100  0.0000              10                            MASS

$*

$* GRID CARDS

$*

GRID*                  1               0-5.000000000E+000.0000000000E+00+

*       2.0000000000E+00               0

GRID*                  2               0-5.000000000E+000.0000000000E+00+

*       -2.000000000E+00               0

GRID*                  3               0-5.000000000E+001.0000000000E+02+

*       -2.000000000E+00               0

$* ELEMENT CARDS

$*

$*  Mesh Collector: Solid(1)

$*  Mesh: 3d_mesh(1)

CTETRA         1       1     296     831     832     833

CTETRA         2       1     544     831     548     833

CTETRA         3       1     549     834     835     833

$* PROPERTY CARDS

$*

$*  Property: PSOLID1

PSOLID         1       1       0                           SMECH

$*

$* MATERIAL CARDS

$*

$*  Material: Steel_2::beam_3D_fem1::[1]

MAT1           12.0600+8        0.3000007.8000-6

$*

$*

$* LOAD AND CONSTRAINT CARDS

$*

$*  Constraint: Fixed(1)

SPC            1       1  123456  0.0000

SPC            1       2  123456  0.0000

SPC            1       5  123456  0.0000

ENDDATA

Theoretical Frequencies Computation

### Flexural vibrations of a beam

The flexural natural frequencies of a beam may be analytically computed using the Euler’s equation:

| EI=∂4v∂x4+ρS∂2v∂t2=0 | () |
| --- | --- |

By solving Eq. (), the formula which allows calculating the natural frequencies based on the imposed boundary conditions is obtained:

| fn=βn22πE⋅Iρ⋅A | () |
| --- | --- |

Where:		E is the Young’s modulus

 is the density;

A is the section area;

I is the moment of inertia;

l is the length of the beam.

In the tables below, the values of the coefficients βnl are reported for two possible cases: clamped beam (Table 1) and free-free beam (Table 2).

| β1l | β2l | β3l | β4l | β5l | β6l | β7l | β8l | β9l | β10l |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,875 | 4,694 | 7,854 | 10,995 | 14,137 | 17,278 | 20,420 | 23,562 | 26,703 | 29,845 |

Table 1: βl value for a clamped beam.

| β1l | β2l | β3l | β4l | β5l | β6l | β7l | β8l |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4,730 | 7,853 | 10,996 | 14,137 | 17,278 | 20,347 | 23,562 | 26,703 |

Table 2: βl value for a free-free beam.

### Torsional vibration of a beam

The torsional natural frequencies of a clamped beam may be analytically computed using the Euler’s equation:

| Ipm∂2θ∂t2=GJ∂2θ∂x2 | () |
| --- | --- |

The formulae needed to calculate the natural frequencies are derived from Eq. (); These frequencies depend on the imposed boundary conditions.

For a beam clamped at one end, the equation becomes:

| fn=2n+1⋅c4⋅l | ( ) |
| --- | --- |

where: 	c=G⋅JIpm

J  Torsional Constant;

Ipm=ρ⋅IpA polar moment of inertia of the mass;

IpA=b⋅h3+h⋅b312 polar moment of inertia of the section;

n = 0, 1, 2, … (the first torsional natural frequency is obtained with n = 0)

Figure 17: torsional mode for a clamped beam.

### Torsional Constant

J is used to designate the Torsional Constant.

Unfortunately, the same variable adopted to denominate the torsional constant is also used to designate the polar moment of inertia of the section (here named IpA). These two quantities are NOT the same. Adding to the confusion, in case of a circular section they are numerically identical:

| IpA=J=π⋅r42 (for a circular section) | () |
| --- | --- |

The polar moment of inertia of the section with respect to an axis OZ (polar axis) perpendicular to the section plane is:

| IpA=r2dm=x2dm+y2dm=Ix+Iy | () |
| --- | --- |

The torsional constant depends on the type of section (Table 3).

| TORSIONAL CONSTANT |  |
| --- | --- |
|  | where = area enclosed by middle line of wall L = entire length of middle line of wall For constant t |
|  | where M is the number of straight or curved segments of thickness ti and width or height bi composing the section, Set a = 1 except as designated otherwise. |

Table 3: Torsional Constant for various types of section.

## Longitudinal vibrations of a beam

The longitudinal natural frequencies of a beam may be analytically computed using this differential equation:

| ∂2u∂t2=Eρ∂2u∂x2 | () |
| --- | --- |

The formula which allows to calculate the natural frequencies based on the imposed boundary conditions is obtained by solving Eq. ().

For a clamped beam the solution is:

| fn=(2n−1)4lEρ | () |
| --- | --- |

where: 	E is the Young’s modulus;

ρis the density

n is the mode;

l is the length.

For a free-free beam the equation becomes:

| fn=n2lEρ | () |
| --- | --- |