FEM TUTORIAL

Software: Simcenter3D

Table of Contents

Intro:

Simcenter3D Basics	3

Shortcuts and useful commands	3

File Management	3

Additional tutorials	4

On-Line Help	4

Exercises:

1.	Modal analysis (SOL 103) of a clamped beam	5

a.	Beam modelled with 1D elements (CBEAM)	5

b.	Alternative method to quickly create a large number of 1D elements	14

c.	Beam modelled with 2D elements (CQUAD)	17

d.	Beam modelled with 3D elements (CTETRA)	21

2.	Forced Response of a Car Door	25

a.	BDF file import and modal analysis	25

b.	Signal in the frequency domain. Direct (SOL 108) and modal (SOL 111) solutions	27

c.	Signal in the time domain. Direct (SOL 109) and modal (SOL 112) solutions	32

3.	Correlation – Gearbox Case	37

# Simcenter3D Basics

## Shortcuts and useful commands

## File Management

In the example above:

“component” 		: master CAD model file (.prt);

“component_fem1_i”	: idealized part (.prt);

“component_fem1”	: FEM file (.fem);

“component_sim1”	: simulation file (.sim).

- Part files (.prt)

Part files contain the CAD geometry. The ‘master’ CAD is the original file (created in Simcenter or imported from another 3D modeling software),  while the idealized part is an additional file created by Simcenter that allows to perform modifications on the geometry without modifying the original file. The generation of the idealized part is optional, and it will not be performed in these tutorials. However, it is reported here for your knowledge, because many tutorials found on the Simcenter website use it.

- FEM file (.fem)

The .fem file contains the meshes, nodes, elements, physical and material properties.

The mesh can be created manually: in this case, a .prt file is not needed. Otherwise, if the mesh is based on a CAD geometry, the .fem file must be associated with a .prt file.

- Simulation file (.sim)

The .sim file contains the solution and all the parameters needed to define it (loads, constraints, boundary conditions and solver-specific data). The mesh is imported from the .fem file.

## Additional tutorials

You can find useful tutorials on the Simcenter website:

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials.html#uid:index_advanced_sim_tutorial

At the end of each exercise proposed in this work, you will find a link to some tutorials chosen from this list. You are suggested to perform them to deepen your knowledge of the software.

## On-Line Help

https://docs.plm.automation.siemens.com/tdoc/nx/1872/nx_help#uid:xid1128419:index_advanced

# Modal analysis (SOL 103) of a clamped beam

The goal is to obtain the first 10 natural frequencies and the corresponding mode shapes of a beam using  1D, 2D and 3D elements. The section of the beam is rectangular, and its dimensions are reported in the table below.

|  | Dimensions [mm] |
| --- | --- |
| Width | 10 |
| Height | 2 |
| Length | 500 |

The goal of this tutorial is to provide all the instruments to:

- Generate a mesh: create nodes and elements, both manually and automatically;

- Set-up a modal analysis: define modal parameters and boundary conditions.

In fact, once the procedure has been understood for 1D elements, it can be generalized for 2D and 3D elements, as the definition of the modal analysis remains the same. As a first approach with the problem, 1D elements are the easiest to work with, thus leading to an easier understanding of the procedure. Later, the same problem will be solved using 2D and 3D elements.

## Beam modelled with 1D elements (CBEAM)

Create a new .fem file. There is no need to create a .prt file, since nodes and elements without an underlying geometry.

- From the starting window, select New  Simulation tab  Simcenter Nastran (fem). In the lower section, define the file name and the folder where the files will be saved.

- In the next window, check if the correct solver (Simcenter Nastran) and analysis type (Structural) are selected.

Now nodes and elements will be manually created.

- Navigate to the Nodes and Elements tab. Select Node Create: the corresponding window will open.

- Insert the coordinates of the nodes by providing the coordinates for each one. In this example, create a beam made of 11 nodes along the Y direction, so that the first node is located at (0,0,0) and the last at (0,500,0). Click Apply to create a node and proceed with the next. Click Ok when all the nodes have been created.

The obtained node distribution should be as the one depicted below:

The next step consists in defining a mesh collector for the elements. Mesh collectors are objects which allow to group meshes and assign physical properties (e.g. materials) to them.

- Navigate to the Home tab. Select Mesh Collector: the corresponding window will open.

- Since the modelling will be done using 1D beam elements, the element family must be set to 1D and the collector type to Beam Collector.

Now, define the physical properties associated to this collector.

- Click on the first button to the right of the row Beam Property: the PBEAML window will open: it is here that the section property and the material must be defined.

- First, create the section. Click on the button next to Fore Section: the beam section manager will open. Select the Create Section button to open the Beam Section window. Here, from the dropdown menu select BAR and define its dimensions in the lines below. Click Ok, then close the Beam Section Manager.

- Now create the material. From the PBEAML window, click the button next to Material: the material list will open. Create a new isotropic material by clicking the button in the bottom right corner: define the density, the Young’s Modulus and the Poisson’s ratio (ρ=7800 kg/m3 , E=206 GPa, ν=0.3). Pay attention to the units of measure! Assign a name to the material and close all.

The collector will appear in the simulation navigator. If needed, its parameters may be edited by right-clicking it and choosing Edit.

Now that the mesh collector is ready, elements may be created.

- Navigate to the Nodes and Elements tab. Select Element Create.

- Set the Element Family to 1D. In Element Properties, choose CBEAM. Under Destination Collector, uncheck Automatic Creation and select from the dropdown menu the name of the previously created collector.

- Create the elements my manually connecting the nodes (select each pair of nodes one after another). To add all the elements to the same mesh, after the first one has been created, select Add to Existing under Destination Mesh.

- The mesh will appear under the associated Beam Collector in the simulation navigator.

Starting from the second element:

The obtained mesh should look as in the figure below:

The orientation of the section must be checked. In fact, the beam axes may not be aligned in the correct way with respect to the reference frame. For instance, in this example the long side of the rectangle which defines the section must be parallel to the X axis.

- First, display the beam sections by superimposing them on the nodes. Right-click on the mesh (found under the Beam Collector) and choose Edit Display. On the Mesh Display window, select Curves under the option Display Section.

- Notice that the section is not aligned as it should be. To modify its orientation, right-click again the mesh under the collector, but this time click Edit Mesh Associated Data. In the Selection Orientation group, in Specify Vector select +ZC as the direction of the Y axis of the section. Confirm everything with Ok.

The .fem part has been completed. Now it will be created the .sim file, in which everything needed to set-up a modal analysis will be defined.

- In the Simulation Navigator, right click the .fem file and select New Simulation. Create a Simcenter Nastran simulation file, choose a name and confirm with Ok. Click Ok again in the next window.

- The Solution window is now open. First, define a name for the solution. Then, to set-up the modal analysis, in the Solution Type line, select SOL 103 - Real Eigenvalues from the dropdown menu.

- Specify the parameters associated with the modal analysis. Under the Case Control tab, choose Lanczos as the Eigenvalue Method. Then, click on the button next to Lanczos Data: the Real Eigenvalue – Lanczos window will open. Under the Frequency Options, three parameters may be defined: lower and upper limits of the analysis frequency range and the number of desired modes. To find the first 10 modes of the system, set Frequency Range – Lower Limit = 0 and Number of Desired Modes = 10. Leave the field Frequency Range – Upper Limit blank. Close with Ok, then close the Solution window: the Solution Step window will open  do not change anything here, simply close it with Ok.

To find the modes of a clamped beam, a fixed contraint needs to be applied to an end of the beam.

- Click the Constraint Type button, and choose Fixed Constraint from the list that will appear. This contraint blocks all the DOFs of the selected node. To select the node, go to the Selection Filter dropdown menu and choose Node (it can be fouid it on the bottom left corner of the top bar, see figure below). Then click and the left-most node to apply the contraint to this end of the beam.

Selection Filter

Selection Filter

Modal parameters and contraints have been defined. Solve the model and examine the natural frequencies of the beam and the corresponding mode shapes.

- To solve the model, select Solve in the top bar (just next to the solution button). Click Ok to start the computation. When the Review Results window appears, click No and close all the other windows that the software has opened.

- To view the results, switch to the Post Processing Navigator. Double click on Structural to load the solution file associated to the just computed solution. The frequency list with the first 10 mode of the system will appear.

- To view the mode shapes, select one frequency, then open the Displacement - Nodal sub-section and double-click on Magnitude.

- To animate the mode, click the Animate button on the top bar. Apply the parameters as you like. Click the Play button in the lower part to start the animation. To see the other modes, use the green arrows in the Result group.

## Alternative method to quickly create a large number of 1D elements

The method described in the previous paragraph is not suitable to model a system with a large number of elements, because each node has to be defined individually by its coordinates. In the case of large models, it is suggested to use the procedure proposed below, which consists in creating a line in the modeling application then meshing it with the 1D mesh command.

- Create a Model. From the starting window, choose New  Model tab  Model. Choose a file name and a folder where the file will be saved.

- This opens the Modeling Section of Simcenter3D. To create a line, navigate to the Curve tab and click on the Line button. In the window that will open, define the start and end point of the line by clicking on the Point Dialog button and providing the coordinates of the points (0,0,0) and (0,500,0). Close the Line window by selecting Ok.

- Create the .fem and .sim files. Navigate to the Application tab, then click on Pre/Post, which opens the corresponding section. Then, click on New FEM and Simulation to create the .fem and .sim files associated with the created part. Uncheck the Create Idealized Part checkbox (the idealized part is not needed in these examples, so it is not created in order to avoid generating useless files).

- To import the line, click on Geometry Option and activate the checkbox Lines. This passage is mandatory, if skipped the line will not appear in the .fem file!

- Since the .sim and the .fem files have been created at the same time, the software will automatically ask to set the solution parameters. The solution may be set up now or it may be defined later by selecting Cancel. Anyway, to set-up the solution, follow the same passage explained in the first example, because the procedure is the same.

[Tip: to switch between the .fem and the .sim files use the Activate Simulation/Activate Meshing buttons located in the Home tab – Context group. Even if the interface looks the same, the commands that are found in these two sections are different!]

- Before creating the mesh, create a mesh collector and define its properties (section area and material) exactly as seen in the previous paragraph.

- To create the mesh, on the Home tab click on the 1D mesh button. On the next window, when asked to select an object click on the line. Under Element Properties select CBEAM. In Mesh Parameters, set Mesh Density by Numbers and Number of Elements (set the number you want, 100 in this example). Choose the previously created Mesh Collector and confirm with Ok.

Once the mesh has been generated, check the section orientation and then proceed to solve the model as in the previous example. Notice that in the first exercise nodes and elements have been manually created one after another, while this time nodes and elements have been automatically generated at the same time by the software.

SIMCENTER TUTORIALS – 1D meshing

- Modeling a beam and cross section

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1677075

- Defining mesh properties with mesh collectors

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1676681

## Beam modelled with 2D elements (CQUAD)

To create a 2D mesh, a surface is needed: it will be created in the modelling application. The dimensions of this surface are 500x10 mm, which are the length and the width of the beam; the height (2 mm) will be set as a property of the section (thickness). Once the surface has been created, it can be easily meshed using the 2D mesh command in the Meshing application. The boundary conditions are applied on all the nodes at one end of the beam, but other than that the procedure to carry out the modal analysis remains the same.

- Create the .prt file. From the starting window, New  Model tab  Model. Define its name and the work folder.

- Create a Sketch. From the Home tab, select Sketch. Click on the XY plane directly on the graphic window to choose it as the sketch plane. If selected correctly, it should appear highlighted in blue, and the view will automatically set normal to it.

- Define a rectangle on the sketch plane. In the Home tab, choose Rectangle. Select the input method of your choice between by 2 points, by 3 points and From Center. If you use by 2 points, set Xc=-5, Yc=0, width=10 and height=500;

Tip: To change a dimension, double-click on its value and change its p0 value.

After the rectangle has been created, close the sketch using Finish Sketch in the Home tab.

- Create the surface. From the Home tab, click on Fill Surface under the Surface group. Select the four lines which define the rectangle: the software fills the inner space defined by these lines and generates the surface.

The surface is ready to be meshed.

- Switch to the Pre/post application. From the Application  tab select Pre/post  New Fem and Simulation. Uncheck the Create Idealized Part box and confirm with Ok.

Notice that this time there is no need to open the Geometry Option window and check the Lines box: in fact, surfaces and bodies are automatically imported in the Pre/post application.

- Define the Solution. In the Solution window, change the Solution Type to Sol 103 - Real Eigenvalue.

- Define the modal parameters. In the Lanczos data section, click on Create Modeling Object. Set Frequency Range – Lower Limit = 0 and Number of Desired Modes = 10. Leave the field Frequency Range – Upper Limit blank. Close the Solution and the Solution Step windows.

- Create a mesh collector. From the Home tab, choose Mesh Collector. In the Element Topology section, set 2D as the Element Family and Thin Shell as the Collector Type. Then, provide the physical properties for the element, which are its material and its thickness. Click on the first button next to Shell Property  to open the PSHELL window (physical properties of the shell elements). Set a material for the Material 1 entry, using the same procedure used to create a material in the 1D model (and using the same values for ρ, E,ν). Eventually, set Default Thickness to 2 mm and close the window.

Material definition

Material definition

- Create the 2D mesh. From the Home tab, select 2D Mesh to open the corresponding window. Select CQUAD as the Element Type and impose Element Size = 2.5 mm, then click the surface on the graphic window to generate the mesh. Remember to choose the mesh collector under the Destination Collector section.

The meshing is completed. Now, apply the boundary conditions and solve the model.

- Switch to the Simulation application by clicking on Activate Simulation on the upper left corner.

- Apply the constraints. Click on Constraint Type  Fixed Constraint. The constraint must be applied to an edge, so from the Selection Filter dropdown menu choose Polygon Edge, then click on the left-most edge of the beam  in this way, a fixed constraint has been created on all the nodes associated to that edge (5 nodes in this example).

- Solve the model. The modal parameters have been defined at the beginning, so everything has been set-up by now. Use the Solve button, then review the results: the commands are the same shown for the 1D model!

SIMCENTER TUTORIALS – 2D meshing

- Creating a 2D mesh

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1676734

- Defining and displaying shell thickness

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1676888

## Beam modelled with 3D elements (CTETRA)

The procedure is essentially similar to the one shown for the 2D model but this time, instead of a surface, a body will be defined and meshed with 3D elements (thus needing a 3D collector, for which the only physical property required is the material).

- Create the .prt file. From the starting window, New  Model tab  Model. Define its name and the work folder.

- Create a Sketch. From the Home tab, select Sketch. This time, create the section on the XZ plane, and then extrude it along the Y direction: click on the XZ plane to specify it as the sketch plane.

- Use the Rectangle command to define the dimensions of the beam section. You may use the method From Center and select the origin as the center, then define width = 10, height = 2 and angle = 0. Close the sketch using Finish Sketch.

- Extrude the section. In the Home tab select the Extrude command. Select the rectangle as the curve to be extruded. Since the chosen sketch plane had its normal oriented in the -Y direction, set a negative value of – 500 mm in the End – Distance field to extrude the section in the positive Y direction.

The part is ready to be meshed.

- Create the .fem and .sim files. From the Application tab  Pre/post  New Fem and Simulation. As for the 2D model, there is no need to modify anything on the Geometry Options window.

- Define the solution. Select Sol 103 – Real Eigenvalues as the Solution Type.

- Define the modal parameters. In the Lanczos data section, click on Create Modeling Object. Set Frequency Range – Lower Limit = 0 and Number of Desired Modes = 10. Leave the field Frequency Range – Upper Limit blank. Close the Solution and the Solution Step windows.

- Create a mesh collector. From the Home tab, choose Mesh Collector. In the Element Topology section, set 3D as the Element Family and Solid as the Collector Type. Now, assign the material. Click on the first button next to Solid Property  to open the PSOLID window (physical properties of the shell elements). Define the material as usual (provide values for ρ, E,ν as described for the 1D model), then close all windows with Ok.

- Create the 3D mesh. From the Home tab, select 3D Tetrahedral to open the corresponding window. Select CTETRA4 as the Element Type and impose Element Size = 1 mm, then click the body on the graphic window to generate the mesh. Remember to choose the mesh collector under the Destination Collector section.

Tip: CTETRA4 elements are linear tetrahedral elements, while CTETRA10 are parabolic tetrahedral elements. In a linear element, nodes are located on the vertices of the tetrahedron  4 nodes element. In a parabolic element, nodes are also located at the midpoint of each edge  10 nodes element.

- Switch to the .sim file (click on Activate Simulation).

- Apply the boundary conditions. Add a fixed constraint at one end of the beam. Use the Selection Filter: choose Polygon Face from the dropdown menu. By selecting the face highlighted below, all the nodes located on that face have been constrained.

- Solve the model and post-process the results.

Tip: for large models, you may experience some graphical lag while reviewing the results. In that case, performance improvement is obtained by using the commands in the Display section to change the visualization (see figure below). This is also useful to make the contours clearer.

SIMCENTER TUTORIALS – 3D meshing

- Using a 2D mesh to seed a 3D mesh

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1676793

- Creating elements without geometry

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1676958

This tutorial shows how to create a 3D mesh only by performing operations on 2D meshes, without the need for a geometry underneath.

- Connecting nodes with spider elements

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1677271

Rigid spiders are elements which allow to rigidly connect multiple nodes using RBE2 elements. They are crucial to simplify a model, e.g. to apply a concentrated mass.

# Forced Response of a Car Door

In this tutorial, the mesh of a car door will be imported in order to perform a forced response analysis.

The forced response will be evaluated:

- By applying a load in the frequency domain and solving using the direct (SOL 108) and modal (SOL 111) solutions.

- By applying a load in the time domain and solving using the direct (SOL 109) and modal (SOL 112) solutions.

The first part (BDF file import and modal analysis) is common for both analysis (frequency and transient). The solution process, on the contrary, will be different since the type of applied load is different.

## BDF file import and modal analysis

- Import the BDF file, which contains the nodes and the elements of the mesh. From the starting window, navigate to the Home tab, then Import Simulation File  select Simcenter Nastran as the solver  import the .BDF file left_door.bdf.

- Have a look at the imported mesh. It is a 2D mesh composed of  CQUAD elements. Also, notice that are some 1D elements.

Basically, the .fem file is already defined. There is only the need to set-up the solution.

- Create the solution. Switch to the .sim file and select Solution from the top bar. Rename the solution, then select SOL 103 Real Eigenvalues as the solution type. Do not close the window yet.

- Define the frequency range. Under Lanczos Data, select Create Modeling Object and define the frequency range for the analysis. Set 0 Hz as the lower limit and 150 Hz as the upper limit  this means that the solver will compute all the modes of the system between 0 and 150 Hz.  Leave the field Number of Desired Modes blank. Close all the windows and proceed. Do not change anything in the Solution Step window which appears, simply accept with Ok.

- Apply the constraints. From the top bar, select Constraint Type and, form the list, choose User Defined Constraint. This constraint allows to restrain only certain DOFs chosen by the user (DOFs 1, 2 and 3 refer to the displacements along the X, Y and Z directions; DOFS 4, 5, 6 refer to the rotations around the X, Y and Z axes): to restrain a DOF, select Fixed for that node in the User Defined Constraint window that will open. Select the nodes 23969 and 23970 (see figure below). Remember to choose Node from the Selection Filter dropdown menu.

- Solve the model. Run the simulation by clicking Solve in the top bar.

- Review the results: check the natural frequencies and animate the mode shapes.

## Signal in the frequency domain. Direct (SOL 108) and modal (SOL 111) solutions

Direct Solution (SOL 108)

- Using the same model used for the modal analysis, create a new Solution. Set the name, then in the Solver Type select SOL 108 Direct Frequency Response.

- Define the forcing frequencies: Case Control  Forcing Frequencies (0)  Create. Define the frequency range where the analysis must be carried out, and the step values at which the results must be obtained, as in the picture below. In this example, frequency response will be evaluated for every integer frequency between 50 and 150 Hz, because the step size has been set to 1 Hz. Be sure not to confuse the forcing frequencies with the loads, as the latter will be defined later: the forcing frequencies are used only to define the exact frequencies for which results are requested.

- In the next window, use the Add       button to add the defined forcing frequency to the solution. If done correctly, Forcing Frequencies (1) should appear in the Case Control section.

- Change the output request so that the software computes the accelerations. Next to Output Requests, click on Edit. Move on the list on the left and choose Acceleration  Enable ACCELERATION Request.

- Close the Solution window and the Solution Step window that opens.

- Define the Constraints. The same procedure described for the modal analysis may be used but, since the same model is being used, a faster method consists in adding the same constraints to the active solution. Open the Constraint Container, right-click the constraint and choose Add to active solution or step. This is an useful command that allows to apply the same constraint to multiple solutions (and it may be used for the loads too, as it will be shown later in another tutorial).

Now define the frequency-dependent load.

- Open the Subcase – Direct Frequency 1 branch.  Right-click Loads and choose New Load Set  Frequency Excitation Set. In the next window, select Field as the Excitation Field, and to define it use the Table Constructor. Remember to check if the excitation type is set to Applied Load (in the upper part).

- Using a table, the excitation curve is defined by its points, i.e. at each frequency is assigned a value to the load. The software linearly interpolates between the points that are provided. In this example, the excitation is a flat spectrum, hence use 4 points to define it.

Before inserting the value, right-click on one column and change the settings to Display Magnitude/Phase (otherwise, the required inputs would be real and imaginary parts of the signal).

Insert the values as in the picture below (frequency, magnitude, phase separated by commas).

This is the curve obtained using these input points:

This only defines the frequency range of the load. Now, it is necessary to define the application point of the load and its direction.

- Open the Frequency Excitation branch, right-click on Load Set and choose New  Force. Apply the load on the node 20444; to orient it in the Z direction, use Specify Vector and choose +ZC. Set Force = 1 N: this value is multiplicated at each frequency by the Frequency Excitation Set (basically, it is a scale factor which depends on the frequency!)  using this procedure, a constant force of 20 N is applied over the range 20 – 200 Hz in the Z direction.

Loads and constraints have been defined: the model is ready to be solved. Note that the frequency range of the forcing frequencies is smaller than the frequency range of the applied load: results will be obtained between 50 and 150 Hz only, even if the range of the applied load is broader!

Once the solution ends, review the results and check the response in a point close to the car door handle.

- Load the results from the Post Processing Navigator. Double-click on one frequency and show its associated Acceleration – Magnitude.

On the Results tab the are several commands that allow to efficiently post-process the results:

- Edit Post view: edit the appearance of the model, including the units of measurement (e.g. to convert mm/s2 to g);

- : use this icon to set the scale of the deformation, or to turn it off;

- Identify Results: assess the requested quantities for the selected nodes and print them on screen;

- Create Graph: select one or more nodes and plot the results over a frequency range.

- Use the Create Graph button to plot the acceleration over the frequency range 50 – 150 Hz. From the upper dropdown menu, choose Across Iterations. When asked to choose a node, arbitrarily choose a node close to the handle, then close with Ok and select the active viewport as the location for the plot.

Modal Solution (SOL 111)

The workflow of the Modal Solution is almost the same of the Direct Solution. In fact, Constraints, Forcing Frequencies, Frequency Excitation Sets and Loads will remain the same. The difference lies in the fact the Damping (critical) and the Modal Parameters necessary to perform the modal solution must be added to solution process.

- First, create a new solution. Proceed as always but choose as solution type SOL 111 Modal Frequency Response.

- Define the Forcing Frequencies. If you are using the same model used for the direct frequency response, the previously created forcing frequencies should be already defined: in this case, simply add them to the list at the bottom.

- Define the modal parameters. Next to Lanczos Data, select Create Modeling Object. In this section, define the lower and upper limits of the frequency, in order to have a sufficiently large modal base for the analysis. As a rule of thumb, the upper limit should be 2 - 2.5 times the value of the highest frequency that needs to be evaluated. In this example, since the highest forcing frequency is 150 Hz, set the frequency range to 0 – 375 Hz.

- Define the damping. In the section Modal Damping, under Damping Type select Critical. On the line below, click on the = and choose New Field  Table. Define a 2 % constant damping for all frequencies using the table constructor, employing the same procedure described for the frequency excitation set. Note that the damping values must be defined as real numbers, not percentages (e.g. to insert 2%, use 0.02). This method is used to define the factor ζ over the frequency range.

- Define loads and constraints by adding the previously defined ones to the active solution.

- Solve the model as usual and post-process the results in the same way as in the previous example.

## Signal in the time domain. Direct (SOL 109) and modal (SOL 112) solutions

The workflow for transient analyses is very similar to the procedure seen for frequency response analyses. In fact, the only differences are that :

- the signal is defined in the time domain instead of the frequency domain;

- the Frequency Excitation Set is replaced by a Transient Excitation Set;

- Forcing Frequencies are replaced by a Time Step definition.

Constraints and loads are applied as usual. Also, for the modal solution, damping and modal parameters are defined in the same way as described for SOL 111.

In this example, the time-dependent excitation will be created using the Table Constructor (exactly as seen for the frequency excitation set), but instead of defining each point one by one in the software, it will be shown an alternative method to import a .txt file which contains all the points.

Direct Solution (SOL 109)

- Define a new Solution: in the Solver Type select SOL 109 - Direct Transient Response.

- Change the Output Request so to enable the ACCELERATION Request. Close the Solution window.

- Define the Time Step: in the Solution Step click on Create Time Step Intervals next to Time Step Intervals (0). In the next window, click on Create and define a time step; in this example, set Number of Time Steps = 125 and Time Increment = 0.002 s. This means that the response of the system will be computed for 125 points, so that the last is at time tend=0.002*125=0.25 s. Remember that is not the load definition, this specifies the instants for which the response will be obtained.

- After Time Step has been defined, use Add       to add it to the current solution. If done correctly, in the Solution Step window Time Step Intervals (0) should have changed to Time Step Intervals (1). Close the Solution Step window.

- Apply the constraints by blocking all DOFs for nodes 23969 and 23970.

Now define the time-dependent load. The procedure is similar to the one used for SOL 108, except that the points are imported from a .txt file.

- Open the Subcase – Direct Transient 1  branch and right-click on Loads, then select New Load Set  Transient Excitation Set.  Click on the = symbol under the property section and select  New Field  Table.

- Set a name for the table, then use the command Import from Text File which is located just above the the Page Control section, under Data Points. Browse to the file transient_event.txt and load it. Its content is not visible on the table: notice that it is defined from 0 s to 0.25 s.

The imported signal is reported on the graph below. According to our time step definition, the response is evaluated at every 0.002 s (i.e. the load applied at each time step is the corresponding red dot).

- Open the Transient Excitation Set branch, right-click on Load Set and choose New  Force. Apply a load equal to 1 N at node 20444, oriented in Z direction (+ZC). Similarly to the Frequency Excitation Set, the Transient Excitation Set works like a time-dependent scale factor for the applied load.

Everything has been set-up and the solution is ready to be computed. Once solved, check the evolution of the response of the system in a node close to the door handle, using the same commands utilized in the post-process of the frequency response; the only difference is that this time the results will be time-dependent instead of being frequency-dependent, as there is one solution for each time increment (as specified in the Time Step definition).

Modal Solution (SOL 112)

The solution procedure for SOL 112 will be reviewed quickly, because it is essentially the same explained for SOL 111 but using the load definition introduced for SOL 109.

- Create a Solution. Select SOL 112 - Modal Transient Response as Solver Type.

- Define the Modal Parameters. In Lanczos Data,  set Lower Limit = 0 Hz and Upper Limit = 375 Hz as the frequency range. Leave the field Number of Desired Modes blank.

- Define the Modal Damping. Select Critical as Damping type, then set a 2 % damping over all frequencies using the Table Constructor (e.g. set 0.02 at 0 Hz and 0.02 at 500 Hz).

- Close the Solution window: the Solution Step window opens.

- Create the Time Step. Use the Create Time Step Intervals button to specify the time steps. Set 125 as the Number of Time Steps and 0.002 s as the Time Increment. Add it to the list for the current solution, then close all windows.

- Apply the Constraints. Use the User Defined Constraint or the Fixed Constraint to block all DOFs for nodes 23969 and 23970.

- Define the Transient Excitation Set. Right-click Loads under the Subcase – Modal Transient 1 and select New Load Set – Transient Excitation Set. Create a table and load the file transient_event.txt using the Import from Text File command.

- Apply the Load. Right-click Load Set under the just created Transient Excitation Set and select New  Force. Apply a 1 N load on the Z direction at node 20444 (Use the Selection Filter to pick the node).

- Solve the model.

- Post-process the results.

# Correlation – Gearbox Case

The goal of this tutorial is to correlate the experimental results of a modal analysis performed on a gearbox case with the numerical results numerically obtained from its FEM model. Correlation is carried out for the first 5 modes of the unconstrained system (i.e. no boundary conditions will be applied).

Experimental modes have been obtained with the LSCE and the polyMAX algorithms: results are stored in the files exp_LSCE.unv and exp_polyMAX.unv. File with extension .unv are used to transfer data between different software. Beside the mode shapes, the .unv files contain the position of the experimental points.

The CAD of the case may be found in the file 61109001b_prt.stp.

Numerical Modes

First, compute the numerical modes using as usual the SOL 103.

- Load the CAD. From the starting window, Open  browse to 61109001b_prt

- Start the Pre/Post application. Navigate to Application  Pre/Post  New Fem and Simulation. Do not generate the idealized part.

- Define the solution. In the Solution window, choose Sol 103 – Real Eigenvalues as the Solver Type

- Define the modal parameters. Under Lanczos data, set Frequency Range – Lower Limit = 1 Hz, and Number of Desired Modes = 5. Leave the field Frequency Range – Upper Limit blank.

This procedure allows to calculate the first five non-rigid modes of the system.

- Close the Solution and Solution Step windows.

- Create a mesh collector. Create a 3D mesh collector, then define its material (Aluminium, E=70 GPa,ρ=2700 kg/m3, ν=0.3).

- Create the 3D mesh. Use the command 3D Tetrahedral, select CTETRA(4) elements and set their dimensions to 2 mm. Assign the mesh to the appropriate mesh collector.

- Solve the model. The system is unconstrained, thus there is no need to apply any boundary condition.

The natural frequencies of the system will be the following:

Natural frequencies and mode shapes needed for the correlation have been computed.

Correlation with LSCE results

- Load the .unv file. Create a new Solution, but this time change the Solver to Modal Test Data. Load the file exp_LSCE.unv both in the Geometry Specifications and Modes File sections.

- After having confirmed with Ok, the 3D mesh will be greyed out. The experimental wireframe may be shown by right-clicking on Test Model 1 under the just created solution and selecting Show: the experimental points are displayed in blue.

- Generate the correlation solution. Display the Correlation tab by right-clicking on the top bar and checking Correlation on the dropdown list which appears. Then, click on the Correlation button: set Test Solution as the Reference Solution (reference = experimental results) and SEMODES 103 as the Work Solution (model to be compared = FEM model). Confirm with Ok.

The correlation solution is finished.

To plot the MAC:

- Navigate to the Correlation tab  Plot MAC  select the current viewport and confirm.

- The rows correspond to the work solution (numerical results).

- The columns correspond to the reference solution (experimental results).

- To read the values in the cells, right-click outside the plot and select Probing Mode  Single XY, then hover the desired cells to see their values .

Numerical

Numerical

Experimental

Experimental

To compare the mode shapes:

- Open the correlation branch (in this is example named correlation_LSCE), then left-click the Mode Pairs [4] under Mode Pairing. Look at the lower part of the simulation navigator, there is a section named Correlation Details View: open it. Inside, there are the mode pairs  right-click on one of them and select Side-by-side Display to display it.

Tip: By default, the software defines as “mode pair” every numerical - experimental pair which has a MAC > 0.7: this way, despite having 5 possible pairs, the software displays only 4 of them. To change the threshold value, right-click on Mode Pairing  Edit  decrease the Minimum MAC to a value of your choice (e.g. set it to 0.6 to view all the pairs in this example).

Correlation with polyMAX results

To correlate the numerical results with the experimental results obtained with the polyMAX algorithm, repeat the same procedure but load exp_polyMAX.unv instead.

SIMCENTER TUTORIALS – Correlation

- Getting started with the correlation solution process

https://docs.plm.automation.siemens.com/tdoc/nx/1872/simcenter_3d_tutorials#uid:index_advanced_sim_tutorial:xid1731225